|  | Room thermostat with LCD TPI control RF set, thermostat with LCD for heating systems, radiators, floor heating for the control of thermal valves, gas or oil boilers or pumps Consisting of thermostat transmitter battery-powered by DC 3 V and thermostat receiver mains-powered by AC 230 V |
| --- | --- |

| Plant diagram |  |
| --- | --- |

|  | Wireless room thermostat with receiver, control of a gas-fired wall-hung boiler |  | Wireless room thermostat with receiver, control of a gas-fired floor-standing boiler |
| --- | --- | --- | --- |
|  | Wireless room thermostat with receiver, control of a heating circuit pump (pre-control by manual mixing valve) |  |  |

|  | F1 | Thermal reset limit thermostat |  | N1 | Room thermostat transmitter |
| --- | --- | --- | --- | --- | --- |
|  | F2 | Safety limit thermostat |  | N2 | Room thermostat receiver |
|  | M1 | Circulating pump |  | Y1 | 3-port valve with manual adjustment |
|  |  |  |  | Y2 | Magnetic valve |

| Function diagram | Heating |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | T | Room temperature |  | Y | Output signal for “Heating” (PWM) |
|  | XpH | Proportional band “Heating” |  | YHC | Control command “Valve” |
|  | w | Room temperature setpoint |  |  |  |

| Description of functions | Sequence |  | Setpoint adjustment |
| --- | --- | --- | --- |
|  | TPI (PWM) control |  | Locally on room thermostat - Turning Knob |
|  | Control of thermal or motorized |  |  |
|  | - valve actuators (On/Off) |  |  |
|  | Heating |  |  |
|  | Room temperature measurement |  |  |
|  | Internal sensor |  |  |

| Device list | Legend; Type of unit; Data sheet; Product No.; Qty.. N1; Wireless room thermostat transmitter with LCD; A6V10954418; RDH100RF (Transmitter); 1. N2; Wireless room thermostat Receiver; A6V10954418; RCR100/433 (Receiver); 1. Electrothermal actuator, AC 230 V, NC, 2P, 1 m; N4884; STA23..; 1. Electrothermal actuator, AC 230 V, NO, 2P, 1 m; N4884; STP23..; 1. <END OptionalDeviceList> |
| --- | --- |
|  | + For selection of sensors, actuators and valves please refer to the product catalog |

| Connection diagram |  | L; Live, AC 230 V. Lx; Live, AC 24…250 V. L1; N.O. contact, AC 24…250 V / 8 (3) A. L2; N.C. contact, AC 24…250 V / 8 (3) A. M1; Circulating pump. N; Neutral conductor. Nx; Neutral conductor. N2; Receiver RCR100/433. Y1; Actuating device. |
| --- | --- | --- |

|  | L – N AC 230 V / Lx – Nx AC 24…250 V |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| Issued by Siemens Switzerland Ltd Building Technologies Division International Headquarters Gubelstrasse 22 CH-6301 Zug Tel. +41 41-724 24 24 www.siemens.com/buildingtechnologies | © Siemens Switzerland Ltd., 2017 Technical specifications and availability subject to change without notice. |
| --- | --- |